SECTION 10440

INTERIOR SIGNAGE

Display hidden notes to specifier by using “Tools”/”Options”/“View”/”Hidden Text”.

** NOTE TO SPECIFIER **  Arris Sign Systems, LLC; modular low-profile sign systems.

.

This section is based on the sign frames of Arris Sign Systems, LLC, which is located at:

398 Grant St. S.E.

Atlanta, GA 30312

Toll Free Tel: (888) 744-6731

Toll Free Fax:  (877) 220-6871

Email: sales@arrissigns.com

Web: http://www.arrissigns.com

Arris Sign Systems is a trade only supplier so its modular, low-profile Arris system is available to sign companies throughout the US and Canada.  With open trade distribution, all the Arris sign solutions are available from your sign vendor of choice.

The Arris system itself is comprised of two unique assemblies, each designed to provide maximum cost effectiveness and design flexibility for specific types of signs.  With both assemblies combined into one comprehensive system, Arris provides ideal solutions for all interior signage needs, from personnel signs and room identification to directories, ceiling signs and kiosks.

Curved face sign products have saturated the market for years as essentially the only viable solution for displaying a wide range of sign materials, notably digitally printed graphics.  Arris provides a well-designed, low profile alternative that does everything a curved product does but with a completely different look.  Able to display a wide range of sign display materials, Arris is designer-friendly yet offers all the functional benefits of a true modular product.

- GENERAL

  - SECTION INCLUDES

** NOTE TO SPECIFIER **  Delete items below not required for project.

    - Interior modular signage system for directional, control, identification and informational signage.

  - RELATED SECTIONS

** NOTE TO SPECIFIER **  Delete any sections below not relevant to this project; add others as required.

    - Section 08110 - Steel Doors and Frames.

    - Section 08210 - Wood Doors.

    - Section 09260 - Gypsum Board Systems.

    - Section 09510 - Acoustical Ceilings.

    - Section 09900 - Painting.

    - Section 09960 - Vinyl Wall Covering.

  - REFERENCES

** NOTE TO SPECIFIER **  Delete references from the list below that are not actually required by the text of the edited section.

    - ANSI A117.1:  Providing Accessibility and Usability for Physically Handicap People, 1986 edition.

    - Department of Justice, Office of the Attorney General, "Americans with Disabilities Act", Public Law 101-336, (ADA).

    - Federal Register Part II, Architectural and Transportation Barriers Compliance Board, 36 CFR Part 1191:  Americans with Disabilities Act (ADA) Accessibility Guidelines for Buildings and Facilities; Amendment to Final Guidelines, September 6, 1991.

    - Federal Register Part III, Department of Justice, Office of the Attorney General, 28 CFR Part 36:  Nondiscrimination on the Basis of Disability by Public Accommodations and in Commercial Facilities, Final Rule, July 26, 1991.

  - SYSTEM DESCRIPTION

    - Signage under this section is intended to include items for identification, direction, control, and information of building where installed as complete integrated system.

    - ADA design requirements:

      - Signage requiring tactile graphics:

        - Wall mounted signs designating permanent rooms and spaces such as, room numbers and restroom, department, office, and fire exit identifications.

        - Individually applied characters are prohibited.

      - Signage not requiring tactile graphics but which are required to comply to other ADA requirements include signs providing direction to or information about function of space such as, directional signs (signs with arrow), informational signs (operating hours, policies, etc.), regulatory signs (no smoking, do not enter), and ceiling and projected wall mount signs.

      - Excluded signage:

        - Exterior signs.

        - Building directories.

        - Menus.

        - Temporary signs, include personnel signs and tenant identification; suite numbers are not considered temporary.

    - ADA performance requirements:

      - Tactile graphics signs mounting requirements:

        - Single doors:  Mount 60 inches (1524 mm) to sign centerline above finish floor and on wall adjacent to latch side of door.

        - Openings:  Mount 60 inches (1524 mm) to sign centerline above finish floor adjacent opening.

        - No wall space adjacent latch side of door, opening, or double doors:  Mount 60 inches (1524 mm) to sign centerline above finish floor on nearest adjacent wall.

  - SUBMITTALS

    - Submit under provisions of Section 01300.

    - Product Data:  Data sheets on each product to be used, including:

      - Product literature indicating units and designs selected.

      - Preparation instructions and recommendations.

      - Storage and handling requirements and recommendations.

      - Installation methods.

    - Samples:  One full size sign sample illustrating the design, construction, colors, typestyles, mounting method and other details as specified. Provide sample in small size sign.

NOTE TO SPECIFIER **  Delete provision not required.

      - Samples will be returned for use in Project.

      - Samples will not be returned for use in Project.

    - Shop Drawings:

      - Indicate materials, sizes, configurations, and applicable substrate mountings.

      - Typography sample for message strips and headers copy.

NOTE TO SPECIFIER **  Option. Delete if not required.

      - Artwork for special graphics.

NOTE TO SPECIFIER **  Option. Delete if not required.

      - Artwork for special headers.

    - Signage Schedule: Complete with location of each sign and the required copy/text.

  - QUALITY ASSURANCE

** NOTE TO SPECIFIER **  Include a mock-up if the project size and/or quality warrant 		taking such a precaution.  The following is one example of how a mock-up on a large 	project might be specified.  When deciding on the extent of the mock-up, consider all the 	major different types of work on the project.

    - Mock-Up:  Sign manufacturer shall provide a mock-up of select signtypes as required for evaluation by the Owner or Architect.  Such sample(s) shall be large enough and complete enough to fully convey the manufacturer’s capabilities and the products’ quality.

1.	Do not proceed with remaining work until workmanship, graphics, and 						installation are approved the Owner or Architect.

    - Installer Qualifications: Trained and authorized for installations of required scope and product.

  - DELIVERY, STORAGE, AND HANDLING

    - Package signs to prevent damage during shipment, handling, storage and installation.  Products are to remain in their original packaging (unless otherwise specified) until removal is necessary for installation.

    - If installation site is not ready for signage upon delivery, store signs in a dry, air-conditioned environment.

    - Handle signage in accordance with manufacturer’s instructions.

    - Store and dispose of solvent-based materials, and materials used with solvent-based materials, in accordance with requirements of local authorities having jurisdiction.

  - PROJECT CONDITIONS

    - Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results.  Do not install products under environmental conditions outside manufacturer's absolute limits.

  - SEQUENCING AND SCHEDULING

    - Schedule system installation after room finishes and fixtures have been completed.

- PRODUCTS

  - MANUFACTURERS

    - Acceptable Frame System Manufacturer:  Arris Sign Systems, LLC; 398 Grant St. S.E., Atlanta, GA 30312.  ASD. Toll Free Tel: (888) 744-6731.  Tel: (404) 688-9000. Fax: (877) 220-6871.  Email: sales@arrissigns.com. Web

- http://www.arrissigns.com.

** NOTE TO SPECIFIER ** Arris System frames (without graphics) are available at a wholesale level to sign manufacturer’s throughout the USA and Canada.  Specify acceptable finished sign manufacture(s) below.

    - Acceptable Sign Manufacturer(s):

** NOTE TO SPECIFIER **  Delete one of the following two paragraphs; coordinate with requirements of Division 1 section on product options and substitutions.

    - Substitutions:  Not permitted.

    - Requests for substitutions will be considered in accordance with provisions of 				Section 01600.

  - ACCEPTABLE PRODUCT

    - Arris low profile modular sign system.

  - SYSTEM REQUIREMENTS

    - General:

      - Sign system shall feature solutions for all required sign types, including but not limited to wall mounted personnel signs, work station personnel signs, primary room 	identification, directories, directionals, overhead signs, projection wall signs, free standing signs, restroom signs, regulatory and information signs, stair signs and changeable slide conference room signs.  All signs within the system must feature the same family of components and convey a uniform look throughout.

    - Features:

      - Updatability: Signs shall allow for easy updating of message inserts without the need to replace the entire sign assembly.  Sign inserts must be front-loaded into the sign frame.

      - No wall mounted signs should feature a depth greater than 11/16 of an inch.

- Wall mounted signs 11 inches wide or less shall feature a maximum depth of

- 7/16 of an inch.

      - Mounting: Signs shall accommodate installation via fully concealed

- mechanical fasteners as required.

    - Graphics and Typography: As selected from manufacturer’s standards.  Reference sign drawings.

    - Colors and Finishes: As selected from manufacturer’s standards.  Reference sign drawings.

    - ADA Compliance: Sign system shall comply with all current and applicable ADA regulations, including requirements regarding which sign types require Braille/tactile features, character heights, color contrast and installation locations and mounting heights within the facility.

    - Materials and Construction:

      - Sign frame assemblies shall be comprised of alloy 6063-T6 extruded aluminum components and a core material of a vinyl polymer, a natural fiber thermoset composite or an aluminum composite rigid enough to support digitally printed graphic displays or other sign insert materials.

      - All aluminum frame extrusions shall contain a minimum of 50% post

- consumer recycled content.

      - Frames shall offer a “green option” using a natural fiber composite cor

- comprised of 50% recycled content.

      - Fasteners: Signs shall be able to accommodate fully concealed mechanical

- fasteners.

  - SIGN SYSTEM COMPONENTS

    - Frames: Select from the following Arris System sign types and configurations as available shown in the Arris System product catalog:

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Desk Signs – Arris Standard Assembly

Frame depth is 7/16 of an inch.  Accommodates sign inserts up to 1/16 of an 					inch thick.

        - 2.5 inches high by 6 inches wide (DS2506)

        - 2.5 inches high by 8.5 inches wide (DS2585)

        - 8.5 inches high by 11 inches wide (DS8511)

        - 8.5 inches high by 8.5 inches wide (DS8585)

        - 11 inches high by 8.5 inches wide (DS1185)

        - Custom Size:  _________________________

        - Frame Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Frame Type:

1)	Standard Assembly – Standard

2)	Standard Assembly – Arris GS Green Solution

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Workstation Signs – Arris Standard Assembly

Frame depth is 7/16 of an inch.  Accommodates sign inserts up to 1/16 of an 					inch thick.  Frame is attached to custom formed aluminum panel clip.

        - 2.5 inches high by 6 inches wide (PC2506)

        - 2.5 inches high by 8.5 inches wide (PC2582)

        - 4 inches high by 4 inches wide (PC0404)

        - 4 inches high by 8.5 inches wide (PC0485)

        - 6 inches high by 6 inches wide (PC0606)

        - 8.5 inches high by 8.5 inches wide (PC8585)

        - Custom Size:  _________________________

        - Frame Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Frame Type:

1)	Standard Assembly – Standard

2)	Standard Assembly – Arris GS Green Solution

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Small Wall Mount Signs – Arris Standard Assembly

Frame depth is 7/16 of an inch.  Accommodates sign inserts up to 1/16 of an 					inch thick.

        - 2.5 inches high by 4 inches wide (WM2504)

        - 4 inches high by 4 inches wide (WM0404)

        - 6 inches high by 4 inches wide (WM0604)

        - 2.5 inches high by 6 inches wide (WM2506)

        - 4 inches high by 6 inches wide (WM0406)

        - 6 inches high by 6 inches wide (WM0606)

        - 7.125 inches high by 6 inches wide with Slider (WM0706SL)

        - 8.25 inches high by 6 inches wide with Bottom Divider (WM0806BD)

        - 2.5 inches high by 8.5 inches wide (WM2585)

        - 4 inches high by 8.5 inches wide (WM0485)

        - 6 inches high by 8.5 inches wide (WM0685)

        - 8.5 inches high by 8.5 inches wide (WM8585)

        - 8.75 inches high by 8.5 inches wide with Bottom Divider (WM8585BD)

        - 9.625 inches high by 8.5 inches wide with Slider (WM9585SL)

        - 11 inches high by 8.5 inches wide (WM1185)

        - 11.25 inches high by 8.5 inches wide w/ Bottom Divider (WM1185BD)

        - 6 inches high by 11 inches wide (WM0611)

        - 8.5 inches high by 11 inches wide (WM8511)

        - 11 inches high by 11 inches wide (WM1111)

        - 17 inches high by 11 inches wide (WM1711)

        - Custom Size:  _________________________

        - Frame Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Frame Mounting:

1)	Vinyl Tape

2)	Mechanical Fasteners

3)	Vinyl Tape + Silicon Adhesive

        - Frame Type:

1)	Standard Assembly – Standard

2)	Standard Assembly – Arris GS Green Solution

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Mini Directories – Arris Standard Assembly

Frame depth is 7/16 of an inch.  Accommodates sign inserts up to 1/16 of an 					inch thick.

        - 7 inches high by 8.5 inches wide with one header and two strips

- (WM0785D2)

        - 9 inches high by 8.5 inches wide with one header and three strips (WM0985D3)

        - 11 inches high by 11 inches wide with one header and four strips (WM1111D4)

        - 13 inches high by 11 inches wide with one header and five strips (WM1311D5)

        - 17 inches high by 11 inches wide with one header and seven strips (WM1711D7)

        - Custom Size:  _________________________

        - Frame Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Frame Mounting:

1)	Vinyl Tape

2)	Mechanical Fasteners

3)	Vinyl Tape + Silicon Adhesive

        - Frame Type:

1)	Standard Assembly – Standard

2)	Standard Assembly – Arris GS Green Solution

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Large Wall Mount Signs – Arris Large Assembly

Frame depth is 11/16 of an inch.  Accommodates 1/8 inch thick sign inserts.

        - 17 inches high by 17 inches wide (WMLA1717)

        - 22 inches high by 11 inches wide (WMLA2211)

        - 22 inches high by 22-1/16 inches wide with two columns (WMLA2222SC)

        - 22 inches high by 33-1/8 inches wide with three columns

(WMLA2233SC2)

        - 22 inches high by 17 inches wide (WMLA2217)

        - 22 inches high by 22 inches wide (WMLA2222)

        - 22 inches high by 28 inches wide (WMLA2228)

        - 24 inches high by 36 inches wide (WMLA2436)

        - 28 inches high by 14 inches wide (WMLA2814)

        - 28 inches high by 28-1/16 inches wide with two columns

(WMLA2828SC)

        - 28 inches high by 42-1/8 inches wide with three columns

(WMLA2842SC2)

        - 28 inches high by 17 inches wide (WMLA2817)

        - 28 inches high by 22 inches wide (WMLA2822)

        - 28 inches high by 28 inches wide (WMLA2828)

        - Custom Size:  _________________________

        - Frame Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Frame Mounting:

1)	Mechanical Fasteners

        - Frame Type:

1)	Large Assembly – Standard

2)	Large Assembly – Arris GS Green Solution

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Directories – Arris Large Assembly

- Frame depth is 11/16 of an inch.  Accommodates 1/8 inch thick sign inserts and removable cartridges of aluminum/paper directory strips.

        - 17 inches high by 11 inches wide with 14 directory strips (DR1711-1)

        - 17 inches high by 22.25 inches wide with 28 directory strips (DR1711-2)

        - 17 inches high by 33.5 inches wide with 42 directory strips (DR1711-3)

        - 17 inches high by 22.25 inches wide with 14 directory strips and one

- display area (DR1711-4)

        - 17 inches high by 33.5 inches wide with 28 directory strips and one display area (DR1711-5)

        - 22 inches high by 11 inches wide with 18 directory strips (DR2211-1)

        - 22 inches high by 22.25 inches wide with 36 directory strips (DR2211-2)

        - 22 inches high by 33.5 inches wide with 54 directory strips (DR2211-3)

        - 22 inches high by 22.25 inches wide with 18 directory strips and one display area (DR2211-4)

        - 22 inches high by 33.5 inches wide with 36 directory strips and one display area (DR2211-5)

        - Frame Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Frame Mounting:

1)	Mechanical Fasteners

        - Frame Type:

1)	Large Assembly

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Header Directories – Arris Large Assembly

- Frame depth is 11/16 of an inch.  Accommodates 1/8 inch thick sign inserts

- and removable cartridges of aluminum/paper directory strips.

        - 22 inches high by 11 inches wide with 14 directory strips and 4 inch

- header (DRH1711-1)

        - 22 inches high by 22.25 inches wide with 28 directory strips and 4 inch

- header (DRH1711-2)

        - 22 inches high by 33.5 inches wide with 42 directory strips and 4 inch

- header (DRH1711-3)

        - 22 inches high by 22.25 inches wide with 14 directory strips, one display

- area and 4 inch header (DRH1711-4)

        - 17 inches high by 33.5 inches wide with 28 directory strips, one display

- area and 4 inch header (DRH1711-5)

        - 27 inches high by 11 inches wide with 18 directory strips and 4 inch

- header (DRH2211-1)

        - 27 inches high by 22.25 inches wide with 36 directory strips and 4 inch

- header (DRH2211-2)

        - 27 inches high by 33.5 inches wide with 54 directory strips and 4 inch

- header  (DRH2211-3)

        - 27 inches high by 22.25 inches wide with 18 directory strips, one

- display area and 4 inch header (DRH2211-4)

        - 27 inches high by 33.5 inches wide with 36 directory strips, one display

- area and 4 inch header (DRH2211-5)

        - Frame Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Frame Mounting:

1)	Mechanical Fasteners

        - Frame Type:

1)	Large Assembly

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Overhead Wall Mount Signs with Horizontal Frame Configuration

- Frame depth is 7/16 of an inch.  Accommodates 1/16 inch thick sign inserts.

        - 8.5 inches high by 17 inches wide (OWM8517(H))

        - 8.5 inches high by 24 inches wide (OWM8524(H))

        - 8.5 inches high by 36 inches wide (OWM8536(H))

        - 11 inches high by 17 inches wide (OWM1117(H))

        - 11 inches high by 24 inches wide (OWM1124(H))

        - 11 inches high by 36 inches wide (OWM1136(H))

        - 11 inches high by 42 inches wide (OWM1142(H))

        - 11 inches high by 48 inches wide (OWM1148(H))

        - Custom Size:  ____________________________

        - Frame Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Frame Mounting:

1)	Mechanical Fasteners

        - Frame Type:

1)	Standard Assembly – Standard

2)	Standard Assembly – Arris GS Green Solution

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Overhead Ceiling Mount Signs with Horizontal Frame Configuration

- Back-to-back 7/16 inch frame assembly.  Accommodates 1/16 inch thick sign

- inserts.

        - 8.5 inches high by 24 inches wide (OCM8524(H))

        - 8.5 inches high by 36 inches wide (OCM8536(H))

        - 8.5 inches high by 48 inches wide (OCM8548(H))

        - 11 inches high by 36 inches wide (OCM1136(H))

        - 11 inches high by 42 inches wide (OCM1142(H))

        - 11 inches high by 48 inches wide (OWM1148(H))

        - 11 inches high by 54 inches wide (OWM1154(H))

        - Custom Size:  ____________________________

        - Frame Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Frame Mounting:

1)	Flush Mount to Ceiling

2)	Wire Suspended from Ceiling

        - Frame Type:

1)	Standard Assembly – Standard

2)	Standard Assembly – Arris GS Green Solution

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Projection Wall Mount Signs.  Back-to-back 7/16 inch frame assembly.

- Accommodates .060 to 1/16 inch thick sign inserts.

        - 6 inches high by 6 inches wide (PM0606)

        - 8.5 inches high by 8.5 inches wide (PM8585)

        - 11 inches high by 8.5 inches wide (PM1185)

        - 11 inches high by 11 inches wide (PM1111)

        - Custom Size:  ____________________________

        - Frame Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Frame Type:

1)	Standard Assembly – Standard

2)	Standard Assembly – Arris GS Green Solution

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Stanchions.	42 inches high aluminum pole assembly with power coated steel

- base plate.

        - 11 inches high by 8.5 inches wide display frame, single-sided

- (FSS1185-SS)

        - 11 inches high by 8.5 inches wide display frame, double-sided

- (FSS1185-DS)

        - 8.5 inches high by 11 inches wide display frame, single-sided

- (FSS8511-SS)

        - 8.5 inches high by 11 inches wide display frame, double-sided

- (FSS8511-DS)

        - 17 inches high by 11 inches wide display frame, single-sided

- (FSS1711-SS)

        - 17 inches high by 11 inches wide display frame, double-sided

- (FSS1711-DS)

        - Stanchion Pole Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Stanchion Base Plate Finish

1)	Silver Power Coated

2)	Other:  __________________________

        - Frame Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

        - Frame Type:

1)	Standard Assembly – Standard

2)	Standard Assembly – Arris GS Green Solution

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Kiosks.

        - 60 inches high by 11 inches wide with 17 inch by 11 inch display area at

- the top and a 42-15/16 inch by 11 inch display area at the bottom of both

- sides of the kiosk.  (FSK6011-D)

        - 60 inches high by 11 inches wide with a 60 inch by 11 inch display area

- on both sides of the kiosk.  (FSK6011)

        - 66 inches high by 17 inches wide with a 24 inch by 17 inch display area

- at the top and a 41-15/16 inch by 17 inch display area at the bottom of

- both sides of the kiosk.  (FSK6617-D).

        - 66 inches high by 17 inches wide with a 66 inch by 17 inch display are

- on both sides of the kiosk.  (FSK6617)

        - 72 inches high by 17 inches wide with a 32 inch by 17 inch display area

- at the top and a 39-15/16 inch display area at the bottom of both sides of

- the kiosk.  (FSK7217-D)

        - 72 inches high by 17 inches wide with a 72 inch by 17 inch display area

- on both sides of the kiosk.  (FSK7217)

        - Kiosk Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Courier Message and Display Strip

        - 4 inch width (CS-04)

        - 6 inch width (CS-06)

        - 8 inch width (CS-08)

        - 8.5 inch width (CS-85)

        - 11 inch width (CS-11)

        - 12 inch width (CS-12)

        - 14 inch width (CS-14)

        - 17 inch width (CS-17)

        - 22 inch width (CS-22)

        - 24 inch width (CS-24)

        - 36 inch width (CS-36)

        - 48 inch width (CS-48)

        - Custom Size:  _________________________

        - Finish:

1)	Natural Satin Anodized

2)	Other:  __________________________

    - Sign Inserts and Graphics:  Select from the following insert types and graphics methods compatible with the Arris system frames:

NOTE TO SPECIFIER **  Delete option(s) not applicable.

      - Engraved Plaques

      - Plaques with Silkscreen Printed Graphics

      - Non-Glare Lens with Subsurface Digitally Printed Graphics

      - 1st Surface Digitally Printed Graphics

      - Laser Printed Directory Strips

      - Tactile/Braille Plaques for ADA Compliance

        - Applied Raised Characters and Raster Braille

        - Photopolymer Characters and Braille

        - Other Comparable and Compliant Tactile Lettering & Braille method.

      - Plaques with Applied Vinyl Graphics

      - Decorative Plaques of Custom Material and Finish(es)

Reference drawings and Signage Schedule for details regarding sign insert 	background colors, copy colors, font specifications, etc. All text and graphics shall be a true representation of typeface(s) and/or graphics specified.

  - FABRICATION

    - Shop assembly:

      - Fabricate units to configurations indicated on reviewed shop drawings.

      - Provide copy on inserts, and covers required on reviewed shop drawings and in accord with ADA requirements.

      - Provide additional blank paper as specified.

      - Include instruction sheets for removal and replacement inserts and installation.

- EXECUTION

  - EXAMINATION

    - Do not begin installation until substrates have been properly prepared.

    - If substrate preparation is the responsibility of another installer, notify Architect of unsatisfactory preparation before proceeding.

  - PREPARATION

    - Clean surfaces thoroughly prior to installation.

    - Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

  - INSTALLATION

    - Install products in accordance with manufacturer’s instructions, in locations and with mounting methods as specified in sign and location drawings.

    - Square, plumb and level all installed products.

    - Install all signage in accordance with the Americans with Disabilities Act Accessibility Guidelines (ADAAG) and any applicable local regulations and/or codes.

    - Upon completion of the work, sign installer shall remove any unused products, materials, packaging and debris from the installation site.

  - CLEANING

    - Clean all exposed surface not more than 48 hours prior to Date of Substantial Completion in accordance with manufacturer’s written cleaning instructions.

  - PROTECTION

    - Protect installed products until completion of project.

    - Touch-up, repair or replace damaged products before Substantial Completion.

  - SIGN SCHEDULES

    - Refer to Signage Schedule and Drawings for sizes, locations, sign types, layouts, typography specifications, sign text/copy and sign graphics.

END OF SECTION